### Prior to Test Date

- Provide instructions to subject

  - proper clothing and shoes

  - food consumption before test

  - time allotment for the test

- Verify with subject

  - time, date, and location

- Check all equipment is working and is calibrated

- Check supplies

- Send subject screening questionnaire (ACSM)

- Send subject waiver form

- Identify if testing if physician should be present

  - If physician needs to be present, send the physician contact information to subject and have them contact the physician.

  - Physician will contact faculty member to arrange test.

- Answer all subject's questions

### On Test day

- Review test procedures with subject and answer all questions

- Subject signs waiver and submits ACSM screening questionnaire.

- Confirm subject meets screening criteria.

  - If Physician is required to be present: the physician will review test procedures and risks with the subject.

- Fill out subject data form

  - Measure height, weight, body composition, etc.

- Invite subject to use the restroom

- Determine age predicted maximum heart rate

- Prepare subject for graded exercise test

  - Fit with appropriately sized mask

  - Fit with heart rate transmitter (e.g., chest strap)

- Inform subject of test procedures

  - demonstrate proper exercise (e.g., treadmill or cycling)

  - Inform subject what to do if he/she decides to stop

      - Show treadmill off button

      - Demonstrate how to dismount treadmill

      - Clarify that he/she can verbally indicate stop at any time

  - Provide instructions on using the Rating of Perceived Exertion (RPE) scale

  - Review how intensity of exercise will change during the test

      - If treadmill: treadmill will progressively increase in speed and/or incline throughout the test at specific time intervals

      - If cycle: resistance of the pedals will increase at specific time intervals

      - The test will last between 10 and 20 minutes

  - blood pressure will be taken throughout the test

  - assure subject they are in control of termination of test

    - They will be encouraged to continue as long as they can, although they will be very tired at the end

    - instruct the subject to signal, or let the tester know if they experience and chest discomfort, dizziness, nausea, severe shortness of breath, extreme leg fatigue and the test will terminate

    - instruct the subject upon termination of the test, he/she will place their hands on the handrail and the treadmill will slow down to a speed where they are able to walk.

  - Ask the subject if they have any questions

### Graded Exercise Test

- Subject seated

  - Begin recording VO2

    - Measure resting VO2 for 5 minutes

    - record resting heart rate

    - take and record resting blood pressure

- Begin test

  - instruct subject during initial stages of test

      - e.g., tell them how much time in the stage before intensity changes

- Every stage

  - ask subject RPE and record

  - take and record blood pressure: 30 seconds before end of each stage

- Before progressing to next stage

  - assess RPE and blood pressure

  - record signs and symptoms of fatigue, dyspnea, or pain

  - ask subject if they can do more work

  - tell subject when workload is going to change

### Terminating the Test

- Termination point of testing

  - subject requests to stop

  - equipment not operating correctly

  - symptom-limited guidelines

  - observed maximal limit reached

  - See ACSM Guidelines for terminating test at the end of this document

      - Record heart rate, blood pressure, RPE

### Recovery

- Cool down

  - 3 to 5 minutes or when heart rate and blood pressure has returned to recommended levels

  - keep subject moving and monitor condition

  - Be cautious, problems often occur during recovery

- Take blood pressure at 1, 3, 5 minutes into recovery

- Note the condition of the patient

- Verbally ensure the subject is doing well and has recovered from the test

- Disconnect the subject

- Advise subject

  - showering, daily activity, avoiding extreme temperatures, appointment for consultation, etc.

### Interpretation and Exercise Prescription

- Calculate VO2 max and METs

- Determine exercise training intensity: heart rate, RPE, or METs

- Review results with subject

ACSM Guidelines for terminating maximal effort graded exercise test

### Absolute Indications

- Suspicion of a myocardial infarction or acute myocardial infarction (heart attack)

- Onset of moderate-to-severe angina (chest pain)

- Drop in systolic blood pressure (SBP) below standing resting pressure or drop in SBP with increasing workload accompanied by signs or symptoms

- Signs of poor perfusion (circulation or blood flow), including pallor (pale appearance to the skin), cyanosis (bluish discoloration), or cold and clammy skin

- Severe or unusual shortness of breath

- CNS (central nervous system) symptoms

  - e.g., ataxia (failure of muscular coordination), vertigo (An illusion of dizzying movement), visual or gait (pattern of walking or running) problems, confusion

- Serious arrhythmias (abnormal heart rhythms)

  - e.g.: second / third degree AV block, atrial fibrillation with fast ventricular response, increasing premature ventricular contractions or sustained ventricular tachycardia)

- Technical inability to monitor the ECG

- Patient's request (to stop)

### Relative Indications

- Any chest pain that is increasing

- Physical or verbal manifestations of shortness of breath or severe fatigue

- Wheezing

- Leg cramps or intermittent claudication (grade 3 on a 4-point scale)

- Hypertensive response (SBP >260 mm Hg; DBP>115 mm Hg)

- Pronounced ECG changes from baseline

  - >2 mm of horizontal or down sloping ST- segment depression, or >2 mm of ST-segment elevation (except in aVR)

- Exercise-induced bundle branch block that cannot be distinguished from ventricular tachycardia

- Less serious arrhythmias (abnormal heart rhythms) such as supraventricular tachycardia