### Submaximal exercise testing

### Prior to Test Date

- Provide instructions to subject

  - proper clothing and shoes

  - food consumption before test

  - time allotment for the test

- Verify with subject

  - time, date, and location

- Check all equipment is working and is calibrated

  - Determine if test is running and/or cycling

  - Determine if the test includes physiological and/or biomechanical measures.

  - Determine submaximal levels to test (e.g., race pace, training pace, uphill, downhill, etc.).

- Check supplies

- Send subject screening questionnaire (ACSM)

- Send subject waiver form

- Answer all subject's questions

### On Test day

- Review test procedures with subject and answer all questions.

- Subject signs waiver and submits ACSM screening questionnaire.

- Confirm subject meets screening criteria.

- Fill out subject data form.

  - Measure height, weight, body composition, etc.

- Invite subject to use the restroom.

- Determine age predicted maximum heart rate.

- Prepare subject for exercise test.

  - If physiological testing:

      - Fit with appropriately sized mask.

      - Fit with heart rate transmitter (e.g., chest strap).

  - If biomechanics testing:

    - Add sensors (e.g., reflective markers, accelerometers, electromyography, etc.).

- Inform subject of test procedures.

  - demonstrate proper exercise (e.g., treadmill, cycling, running overground).

  - Inform subject what to do if he/she decides to stop.

      - Show treadmill off button.

      - Demonstrate how to dismount treadmill.

      - Clarify that he/she can verbally indicate stop at any time.

  - Provide instructions on using the Rating of Perceived Exertion (RPE) scale.

  - Assure subject they can stop the test at any time.

  - Ask the subject if they have any questions.

  - Review test conditions (e.g., pace, intensity, time, number of conditions, etc.) with subject.

  - Describe to the subject all measures that will be taken during the test day.

### Test

  - Measure resting parameters while subject is seated.

  - If physiological measures are included:

    - Begin recording VO2 (while subject is seated)

    - Measure resting VO2 for 5 minutes

  - Begin exercise portion of test

    - Gradually increase intensity until the target intensity is reached.

    - Maintain this intensity for 5-10 minutes while measurements are made.

      - Measure VO2, HR, RPE, etc.

      - If lactate testing: Finger prick for each running condition

  - Repeat process for each condition or trial.

### Recovery

- Cool down

    - 3 to 5 minutes or when heart rate has returned to within 20 beats per minute of resting.

- Disconnect the subject

- Review results with subject