Lab Title: Kinetic Analysis of Isometric Calf Raise

Objective

- Measure and analyze Isometric Calf Raise using force frames to determine key metrics such as peak force and rate of force development (RFD).

- Learning Outcome: Participants will understand the differences between their left and right legs in terms of power output from their calves.

Background Information

- An Isometric Calf Raise involves the participant sitting with their knee of interest directly under a pad from the force frames, in which they will drive that knee up into the pad and hold it for 3 seconds with maximal effort.

- This testing is used to assess calf power, neuromuscular coordination and fatigue, and force production capabilities, making it a valuable test for athletes and research in sports performance.

Key Terminology

- Peak Force: The highest amount of force generated during the calf raise

- Rate of Force Development (RFD): the rate at which force is developing, can be analyzed in different phases.

Instruments & Equipment

- Stool with adjustable height

- VALD Force Frames

- Tablet or Laptop: Vald Software ready and account created

- Measuring Tape

Procedures

Preparation:

- Record measurements of lower body (leg segment, thigh segment & shin segment)

  - Measurement Sites:

    - Leg Segment- greater trochanter to lateral malleolus

    - Thigh Segment - greater trochanter to lateral epicondyle

    - Shin Segment - lateral epicondyle to lateral malleolus

- Familiarize participants with proper calf raise technique

- Calibrate and set up the VALD force platform according to manufacturer guidelines.

- Ensure that the participant wears suitable athletic footwear and warms up appropriately.

Testing Protocol

- Participant will sit on a stool and position the knees underneath the pads and feet flat on the ground

  - Right knee under right pad, left under left pad.

- Force Frame height must be adjusted to right above knee, it cannot be touching the participant before the trial begins

- The participant's thighs should be parallel to the ground, knee joint approximately 90 degrees with a vertical shin angle when sitting.

  - Adjust stool height if necessary

- Participant’s hands must be on their hips, across their chest, or behind their head to eliminate using extra parts of body for support

- On your cue, the participant will perform a calf raise pushing off the ball of their foot while simultaneously driving their knee into the pad and hold it with maximal effort

  - Do one leg at a time

- Allow for 3 seconds of data collection.

- Have the participant perform another trial on the same leg

- Perform the same procedure with the other leg

Data Collection:

- Collect data from the force frame

- Record the key metrics Peak Force and RFD, all of which VALD automatically records and saves.

Results

- Summarize calf raise data for the participant

- Compare results between the left and right trials

Discussion Questions

- Was one calf significantly stronger than the other? Why do you think that may be?

- Was one calf significantly quicker at getting to peak force than the other? Why?

- Do you think the differences in segment length played a role in testing outcome? Why?

- Referring to the last question, do you think an individual with a longer thigh segment or shin segment would have an advantage when performing this test?

Conclusion

- Summarize the findings of the calf raise lab, highlighting the relationship between force production and RFD, as well as how the legs force production and RFD differed for this trial.

- Discuss how athletes might use knee drive data could help develop their training progression and potentially set a baseline for return to play athletes.

Future Applications

- Explore how metrics collected during the isometric calf raise can be integrated into individualized training programs for performance programming.

- Consider longitudinal tracking of calf raise performance as a measure of training progress and readiness for competition.

Safety & Ethical Considerations

- Ensure proper warm-up to reduce injury risk, especially in explosive movements.

- Clean area before use to avoid any tripping or slipping on outside materials.